CV (coefficient of velocity) formula and example:

|  | Theory | 125% of | Each | Total |
| --- | --- | --- | --- | --- |
| # Act |  |  |  | 1 |
| Bore |  | 1.997054408 | 2 |  |
| Area | 2.505882353 | 3.132352941 | 3.1416 |  |
| Stroke |  |  | 4 |  |
| Volume |  |  | 12.5664 | 12.5664 |
| PSIG |  |  | 85 |  |
| CF |  |  | 6.782312925 |  |
| Delta P |  |  | 8.5 |  |
| PSIG 2 |  |  | 76.5 |  |
| Force |  |  | 213 | 213 |
| Time |  |  | 1.25 |  |
| SCFM |  |  |  | 2.367479365 |
| CV |  |  |  | 0.087072152 |

To find CV (coefficient of velocity) we first must know some variables to find it. All the green boxes above except the green bore box are variables that we will need to know before we can start the formulation of CV.

- Information we need to find out:

  - Total number of actuators

  - Stroke length of one actuator in inches

  - PSIG that we can expect to see at the actuator

  - Total force that the actuators will be moving in pounds

  - Time that the actuator is expected to move the force or load in seconds

- Step # 1

  - Enter all needed variables in their appropriate boxes

- Step # 2

  - Find the force for each actuator

    - Each=Total Force of Actuators

- Step # 3

  - Find the theoretical area

    - Area=Force of eachPSIG

- Step # 4

  - Find 125 % of the area

    - 125%=Theory Area×1.25

- Step # 5

  - Find the 125 % bore size

    - Diameter=125% Area.7854

- Step # 6

  - If the 125% bore size does not fall exactly on a standard bore size move up to next bore size larger

    - Enter into the “Bore” size “Each” box

- Step # 7

  - Find the area for the standard bore size

    - Area=Diameter2×.7854

- Step # 8

  - Find the volume for each actuator

    - Volume=Area of Each×Stroke

- Step # 9

  - Find the total volume

    - Total Volume=Volume of Each ×  of Actuators

- Step # 10

  - Find CF (compression factor)

    - CF=(PSIG+14.7)14.7

- Step # 11

  - Find Delta P, “ P”, is 10% of PSIG

    - ΔP=PSIG×.1

      - 0.1 is a constant

- Step # 12

  - Find PSIG 2

    - PSIG2=PSIG−ΔP

- Step # 13

  - Find SCFM, (standard cubic feet per minute)

    - SCFM=(Total Volume × CF)(Time ×28.8)

      - 28.8 is a constant, is the cubic inches per second

      - 28.8=1728 cu in60 Seconds

- Step # 14

  - Find CV (coefficient of velocity)

    - CV=(SCFM × 1.024)(Δp ×(PSIG2 + 14.7)

      - 1.024 is a constant

- Step # 15

  - Find the desired valve series and port size from the manufacturers catalog. Match the CV with the given CV or chose the closest CV above the calculated CV.