Showport - Shows the status of link port lines

Legal junk:

Copyright 1997 Shawn D'Alimonte

This program is freeware.  It may be used and copied as long as no charge

other than reasonable charges for disks, etc. are made.

If you make changes to this program than please give me credit and indicate

exactly hwat you have changed.

Seeing as this program is free, it comes with no warrenty of any kind.  If it

doesn't work for you than it's not my problem.  Backup your calculator before

using this program, just in case.

OK now for the real docs:

I was writing some link software for my Amiga (aminet:Alink85.lha) when I

noticed how nice it would be to be able to see what the link port lines were

while getting the low level port routines right.  I was getting tired of useing

a multimeter and figure the calculator could do the job.  This is the result.

When run this program indicates the status of the two lines of the link port

by two lines on the display.  The one above the T is for the tip of the link

connector (Red wire) and the one above the R is for the middle part of the plug

(White wire).  When the bar is showing that means that that port line is

receiveing a high signal(~5V) while no bar means it is getting a low

signal(~0V).  Press [EXIT] when you are done/bored with it.

This is my first real ZShell program and my first non-trivial program in any

assembler language so don't expect great style or efficiency.

Send all comments/suggestions to sdalimon@ee.ryerson.ca or aa600@torfree.net

My other program is ALink85 which is a link program for the TI-85 and the $4

serial link for the Amiga.  Get it from AmiNet(http://wuarchive.wustl.edu/aminet)

I wrote this program using asz80 from the Frankenstien package of cross

assemblers found on AmiNet.  Unfortunatly it has some differences from TASM

and does not support macros so the code is a little different looking than most

ZShell code written for TASM.

Other stuff used to make it:

MEmacs	- one of the editors that comes with the Amiga.  A stripped down Emacs

asz80	- mentioned above.  One the 3 or 4 z80 assemblers I found on Aminet

no macro support, but otherwise pretty good.  From the Frankenstein

package.

hex-bin	- some source code I found to turn IHex from the assembler into binary

string85- homemade version since no one seems to have released source code for

this necessary utility

and a shell script to run the assembler and other programs to automate the

.asm->.85s process

If you want hex-bin, string85 and/or my script just mail me.