KyleCorp.

TI83+ Experimental Color Screen

Code Name “The RGB Screen”

I have designed a circuit that will flash a red, blue and green light LED at different intervals and will be controlled by an ASM program on a TI83+ or other compatible calculator.  This is a hardware add-on that will be placed next to or under a TI83+ compatible LCD screen and is connected to the linkport. The included files in this zip file are:

ION (Folder)

RGBION.8xp

RGBION.z80

MirageOS (Folder)

RGBMOS.8xp

RGBMOS.z80

TIOS (Folder)

BOOT.8xp

BOOT.txt

RGBTIOS.8xg

RGBTIOS.8xp

RGBTIOS.z80

KyleCorp.TK (Internet Shortcut for those that can reach .TK Domains)

KyleCorp Homepage Direct Link (Internet Shortcut for those that can’t reach .TK Domains; Subject To Change)

KyleCorp Forum (Internet Shortcut)

KyleCorp RGB Screen.pdf

KyleCorp RGB Screen.wpd

KyleCorp The RGB Screen PCB.jpg

KyleCorp The RGB Screen PCB Layout.jpg

KyleCorp The RGB Screen SCH.jpg

RGBDrive.inc

Universal Parts List:

1 2.5mm stereo audio jack

2 3v button cell type batteries (like the CR2032)

2 3v button cell type battery holder (To house the 2 batteries)

1 0.1 µF Capacitor

3 2.2K Ω ¼ Watt Resistors

6 BC547B NPN Transistors or equivalent (make sure you have the pinouts correct!)

2 74AC00PC Quad 2-Input NAND Gate IC’s or equivalent

1 Super Bright High Intensity Non-Diffused Red LED

1 Super Bright High Intensity Non-Diffused Blue LED

1 Super Bright High Intensity Non-Diffused Green LED

1 Red Colored LED Holder

1 Blue Colored LED Holder

1 Green Colored LED Holder

1 Copper Clad Board

Solder

Mouser Parts List:

161-2505

658-CR2032

593-4000G

593-4000B

593-4000R

604-L7113PBCH

638-333-2SURCS5306

638-383-2SUGCS400

614-SMTU2032-1

512-74ACT00PC

71-CCF07-G-2.2K

512-BC547BTFR

(You provide your own solder and copper clad board)

The schematic is shown below:

The PCB is shown below:

Note:  Do not mirror the image prior to printing; just print the image (it’s already scaled correctly), transfer it to your copper clad board, drill & etch, and attach the components to the side opposite the copper traces (except for the battery holders; they are SMT battery holders so they are soldered onto the same side where all of the copper traces are.) Use KyleCorp The RGB Screen.jpg image to etch the board; use the picture above as a guide to where you should place the components. If you want, you can also attach this to your slide case so that when you put your calculator in your slide case ready for use the LED’s will hang over the top of the screen.  Once you fully assemble The RGB Screen, then you can affix this label to your calculator’s battery cover:

There are pictures of the completed circuit on my website at www.kylecorp.tk or www.agility.frandt.com/kylecorp.

The RGB Screen is rather simple once it is setup hardware wise.  First, you send "RGB.8xg" to your calc's Archive and from there you ungroup it to RAM and run BOOT.8xp from TIOS or MirageOS or send “RGBMOS.8xp” to RAM and run it from MirageOS or you can send “RGBION.8xp” to RAM and run it from an Ion Compatible Shell.  The Include File (RGBDrive.inc) and the Source Codes (BOOT.txt, RGBTIOS.z80, RGBMOS.z80, and RGBION.z80) are all self-explanatory.  If you will use my source code in any way you must give me credit for it.  To turn on the Red LED, the tip is set high and the ring is set low.  To turn on the Blue LED, the tip is set low and the ring is set high.  To turn on the Green LED, the tip and the ring of the linkport are both set high.

Truth Table for the Inhibited AND Logic Gate (Connected to Red LED):

| Input A | Input B | Output B’ | Output Z |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 |

Truth Table for the Inhibited AND Logic Gate (Connected to Blue LED):

| Input A | Output A’ | Input B | Output Z |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |

Truth Table for the AND Logic Gate (Connected to Green LED):

| Input A | Input B | Output Z |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

7 colors are supported: Red, Green, Blue, Yellow, Cyan, Magenta, and White.  As we all know “Black Light” does not exist because black is the absence of light.  However I will add support for this novelty “Black Light” in version 1.7 of The RGB Screen. The truth table for that is:

| Red LED | Green LED | Blue LED | Output Color |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Black |
| 0 | 0 | 1 | Blue |
| 0 | 1 | 0 | Green |
| 0 | 1 | 1 | Cyan |
| 1 | 0 | 0 | Red |
| 1 | 0 | 1 | Magenta |
| 1 | 1 | 0 | Yellow |
| 1 | 1 | 1 | White |

Now for some screenshots of the completed RGBScreen:

The two CR2032 batteries are the power supplies for the two logic gate IC’s.  The Source code for the TI83+ is:

The Source Code(s) can be adapted to probably all TI calculators, and pretty much anything that can send high low signals. If you have questions or comments please visit KyleCorp.TK for more information (for those of you who can't connect to a .tk domain, use http://agility.frandt.com/kylecorp instead.) A forum is located at http://agility.frandt.com/forumboard for those of you who would like to ask questions and talk about this product.  That will be the Official Forum Board for The RGB Screen project.  I would like to thank my Dad for teaching me all of the electronics skills I needed to make this project possible and to all of those who have written online tutorials you help people like me accomplish great things.  Thank you all.  This has been a development of KyleCorp.

-Kyle Bond

Chief Executive of KyleCorp.