Appendix A

Schematic overview

| Writing |  | Reading |  | Hardware |  |
| --- | --- | --- | --- | --- | --- |
| $D0 | %1101.0000 | $C | %0000.1100 | Tip = +5v | Ring = +5v |
| $D1 | %1101.0001 | $8 | %0000.1000 | Tip = +0v | Ring = +5v |
| $D2 | %1101.0010 | $4 | %0000.0100 | Tip = +5v | Ring = +0v |
| $D3 | %1101.0011 | $0 | %0000.0000 | Tip = +0v | Ring = +0v |

Some useful equates:

# For those of you who hate to work with bits and numbers. Although I personally prefer to stay as close to the bits as I can, to keep as much control as I can. I’m also one of those few who still build websites by writing HTML. This is the same as “linkport.inc” in “Code examples and software”:

; *************************

; CORRECT LINK PORT EQUATES

; *******By Timendus*******

;

; Usage: #include "linkport.inc"

;

; * Writing to the link port:

;

;	ld a,Set_TipLow_RingLow

;	out (Linkport),a

;

; * Reading from the link port:

;

;	in a,(Linkport)

;	and Read_Tip

;	or a		; Equal to "cp 0"

;	jr z,Tip_is_low

;	jr nz,Tip_is_high

;

; * Or:

;

;	in a,(Linkport)

;	bit Read_TipB,a

;	jr z,Tip_is_low

;	jr nz,Tip_is_high

Linkport			.EQU	$0

Set_TipHigh_RingHigh	.EQU	$D0

Set_TipLow_RingHigh	.EQU	$D1

Set_TipHigh_RingLow	.EQU	$D2

Set_TipLow_RingLow	.EQU	$D3

Read_Tip			.EQU	$4

Read_Ring			.EQU	$8

Read_Both			.EQU	$C

Read_TipB			.EQU	2

Read_RingB			.EQU	3