# Appendix B

Original Texas Instruments documentation

(If you wish to use “D0LD1L” etcetera, you’ll have to edit your Ti83asm.inc, since the corresponding values are incorrect. See “Linkport Tutorial.doc” for more information.)

TI83 LINK PORT

--------------

This brief look at the TI-83 link port shows how to access

the port and use it for basic sending and receiving of bytes of

data.

The TI-83 link port uses 2 data lines, D0 & D1, for communicating.

These data lines are accessed through the B-port of the Z80.

The symbol BPORT is equated to the correct port value in

the include file "TI83ASM.INC".

Note :  THE DATA LINES ARE READ IN AS HIGH (1) WHEN NO

ACTIVITY IS PRESENT ON THE DATA LINES.

THE LOWER 2 BITS OF THE B-PORT, BITS 0 AND 1, ARE FOR

WRITING TO THE DATA LINES.

BITS 2 AND 3 OF THE B-PORT ARE FOR READING IN THE STATUS

OF THE DATA LINES.

Reading the data lines :

IN      A,(BPORT)       		; READ THE VALUE OF THE B-PORT

CP      0Ch             		; ANY DATA LINE GO LOW ?

JR      Z,NO_ACTIVITY   	; IF READ 0Ch THEN NO LINES LOW

CP      8               		; IS D0 PULLED LOW ?

JR      Z,D0_LOW       	; YES, BIT 2 = 0 SO D0 IS LOW NOW

;

; ELSE BIT 3 WAS LOW (ACC=4), SO D1 IS LOW

;

Writing to the data lines :

There are symbols equated to the correct values

to "OUT" to the B-PORT for controlling the

status of the data lines.

LD      A,D0LD1L

OUT     (BPORT),A    		; is used for setting D0 low, D1 low

LD      A,D0LD1H

OUT     (BPORT),A    		; is used for setting D0 low, D1 high

LD      A,D0HD1L

OUT     (BPORT),A    		; is used for setting D0 high, D1 low

LD      A,D0HD1H

OUT     (BPORT),A   		; is used for setting D0 high, D1 high

A few of the TI-83 system routines concerning the link port

are available for ASM use. These routines are accessed by making

a call to the routine _IO_EXEC, with a value stored in the

byte at ASM_IND_CALL (80C8h).

_IO_EXEC = 51EFh

D0LD1L	EQU	0C3h

D0LD1H	EQU	0C1h

D0HD1L	EQU	0C2h

D0HD1H	EQU	0C0h

BPORT	EQU	0

ASM_IND_VALUE

-------------

19d         REC1STBYTE : This routine goes into idle

or low power mode and waits for

the data lines to change. Then

reads a byte of data using the TI-83

bit protocol. This only reads the 1st

byte of data. The byte is returned

in the accumulator.

20d         REC1STBYTENC : The same as REC1STBYTE except

that the cursor does not flash.

The byte is returned in the

accumulator.

22d         RECABYTE     : This routine looks at the data

lines for activity for about

2 seconds and reads in a single

byte of data. If no data is found

an error will be generated.

The byte is returned in the

accumulator.

11d         SENDABYTE    : This routine sends a byte of

data across the data lines

using the TI-83 bit protocol. If

there is no response within about 2

seconds an error is generated.

The data to be sent needs to be in

the accumulator.