Single byte Z80 OpCodes

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Nop | Djnz * | Jr nz, * | Jr nc, * | Ld b, b | Ld d, b | Ld h, b | Ld (hl), b | Add a, b | Sub b | And b | Or b | Ret nz | Ret nc | Ret po | Ret p | 0 |
| 1 | Ld bc, * | Ld de, * | Ld hl, * | Ld sp, * | Ld b, c | Ld d, c | Ld h, c | Ld (hl), c | Add a, c | Sub c | And c | Or c | Pop bc | Pop de | Pop hl | Pop af | 1 |
| 2 | Ld (bc), a | Ld (de), a | Ld (*), hl | Ld (*), a | Ld b, d | Ld d, d | Ld h, d | Ld (hl), d | Add a, d | Sub d | And d | Or d | Jp nz, * | Jp nc, * | Jp po, * | Jp p, * | 2 |
| 3 | Inc bc | Inc de | Inc hl | Inc sp | Ld b, e | Ld d, e | Ld h, e | Ld (hl), e | Add a, e | Sub e | And e | Or e | Jp * | Out (*), a | Ex (sp), hl | Di | 3 |
| 4 | Inc c | Inc d | Inc h | Inc (hl) | Ld b, h | Ld d, h | Ld h, h | Ld (hl), h | Add a, h | Sub h | And h | Or h | Call nz, * | Call nc, * | Call po, * | Call p, * | 4 |
| 5 | Dec b | Dec d | Dec h | Dec (hl) | Ld b, l | Ld d, l | Ld h, l | Ld (hl), l | Add a, l | Sub l | And l | Or l | Push bc | Push de | Push hl | Push af | 5 |
| 6 | Ld b, * | Ld d, * | Ld h, * | Ld (hl), * | Ld b, (hl) | Ld d, (hl) | Ld h, (hl) | Halt | Add a,(hl) | Sub (hl) | And (hl) | Or (hl) | Add a, * | Sub * | And * | Or * | 6 |
| 7 | Rlca | Rla | Daa | Scf | Ld b, a | Ld d, a | Ld h, a | Ld (hl), a | Add a, a | Sub a | And a | Or a | Rst 00h | Rst 10h | Rst 20h | Rst 30h | 7 |
| 8 | Ex af, af' | Jr * | Jr z, * | Jr c, * | Ld c, b | Ld e, b | Ld l, b | Ld a, b | Adc a, b | Sbc a, b | Xor b | Cp b | Ret z | Ret c | Ret pe | Ret m | 8 |
| 9 | Add hl, bc | Add hl, de | Add hl, hl | Add hl, sp | Ld c, c | Ld e, c | Ld l, c | Ld a, c | Adc a, c | Sbc a, c | Xor c | Cp c | Ret | Exx | Jp (hl) | Ld sp, hl | 9 |
| A | Ld a, (bc) | Ld a, (de) | Ld hl, * | Ld a, (*) | Ld c, d | Ld e, d | Ld l, d | Ld a, d | Adc a, d | Sbc a, d | Xor d | Cp d | Jp z, * | Jp c, * | Jp pe, * | Jp m, * | A |
| B | Dec bc | Dec de | Dec hl | Dec sp | Ld c, e | Ld e, e | Ld l, e | Ld a, e | Adc a, e | Sbc a, e | Xor e | Cp e |  | In a, (*) | Ex de, hl | Ei | B |
| C | Inc c | Inc e | Inc l | Inc a | Ld c, h | Ld e, h | Ld l, h | Ld a, h | Adc a, h | Sbc a, h | Xor h | Cp h | Call z, * | Call c, * | Call pe, * | Call m, * | C |
| D | Dec c | Dec e | Dec l | Dec a | Ld c, l | Ld e, l | Ld l, l | Ld a, l | Adc a, l | Sbc a, l | Xor l | Cp l | Call * |  |  |  | D |
| E | Ld c, * | Ld e, * | Ld l, * | Ld a, * | Ld c, (hl) | Ld e, (hl) | Ld l, (hl) | Ld a, (hl) | Adc a, (hl) | Sbc a, (hl) | Xor (hl) | Cp (hl) | Adc a, * | Sbc a, * | Xor * | Cp * | E |
| F | Rrca | Rra | Cpl | Ccf | Ld c, a | Ld e, a | Ld l, a | Ld a, a | Adc a, a | Sbc a, a | Xor a | Cp a | Rst 08h | Rst 18h | Rst 28h | Rst 38h | F |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |  |

Single bytes are taken as-is except the case where you have a "*" which means the number you put after it will be appended to the hex code

The star may take one or two additional bytes depending if you're accessing a 8 or 16bit register denotes by the number of letters (a=8, bc=16)

If you wish to know the code for a Ix type, 				append "0DDh" after the first OpCode.

If you wish to know the code for a Iy type, 				append "0FDh" after the fisrt OpCode.

If you have either Ix or Iy in the command,				append one byte right after the second byte to give the offset (-128..127)

If you are working at a bit level (ie : bit, set, res, etc...), 		append "0CBh" after the first OpCode and add (8 * bit) to the first byte

In the case where you have either Ix, Iy and working with bits	Let the first byte be "0CBh" and follow the previous rule for Ix, Iy (reversed)

For the other commands which are not any of the above,		append "0EDh" after the fisrt OpCode.

Multibytes Z80 OpCodes

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Rlc b | Rl b | Sla b | In b, (c) | Bit *, b | In d, (c) | In h, (c) | ld i?h, b Ld (i?), b | Res *, b |  | Ldi | Ldir | Set *, b |  |  |  | 0 |
| 1 | Rlc c | Rl c | Sla c Ld i?, * |  | Out (c), b Bit *, c | Out (c), d | Out (c), h | Ld i?h, c Ld (i?), c | Res *, c |  | Cpi | Cpir | Set *, c |  | Pop i? |  | 1 |
| 2 | Rlc d | Rl d | Sla d Ld (*), i? |  | Sbc hl, bc Bit *, d | Sbc hl, de | Sbc hl, hl Ld i?h, d | Sbc hl, sp Ld (i?), d | Res *, d |  | Ini | Inir | Set *, d |  |  |  | 2 |
| 3 | Rlc e | Rl e | Sla e | Inc i? | Ld (*), bc Bit *, e | Ld (*), de | Ld i?h, e | Ld (*), sp Ld (i?), e | Res *, e |  | Outi | Otir | Set *, e |  | Ex (sp), i? |  | 3 |
| 4 | Rlc h | Rl h | Sla h | Inc (i?) | Ld b, i?h Bit *, h | ld d, i?h | Ld i?h, h | Ld (hl), i?h | Res *, h Add a, i?h | Sub i?h | And i?h | Or i?h | Set *, h |  |  |  | 4 |
| 5 | Rlc l | Rl l | Sla l Dec i?h | Dec (i?) | Ld b, i?l Bit *, l | Ld d, i?l | Ld i?h, l | Ld (hl), i?l | Res *, l Add a, i?l | Sub i?l | And i?l | Or i?l | Set *, l |  | Push i? |  | 5 |
| 6 | Rlc (hl) | Rl (hl) | Sla (hl) Ld i?h, * | Ld (i?), * | Im 0 Bit *, (hl) | Im 1 ld d, (i?) | Ld i?h,(hl) Ld h, (i?) |  | Res *, (hl) Add a,(i?) | Sub (i?) | And (i?) | Or (i?) | Set *, (hl) |  |  |  | 6 |
| 7 | Rla a | Rl a | Sla a |  | Ld i, a Bit *, a | Ld a, i | Rrd ld i?h, a | Ld (i?), a | Res *, a |  |  |  | Set *, a |  |  |  | 7 |
| 8 | Rrc b | Rr b | Sra b | Srl b | In c, (c) | In e, (c) | In l, (c) ld i?l, b | In a, (c) |  |  | Ldd | Lddr |  |  |  |  | 8 |
| 9 | Rrc c Add i?, bc | Rr c Add i?, de | Sra c Add i?, i? | Srl c Add i?, sp | Out (c), c | Out (c), e | Out (c), l Ld i?l, c | Out (c), a |  |  | Cpd | Cpdr |  |  | Jp (i?) | Ld sp, i? | 9 |
| A | Rrc d | Rr d | Sra d Ld i?, (*) | Srl d | Adc hl, bc | Adc hl, de | Adc hl, hl ld i?l, d | Adc hl, sp |  |  | Ind | Indr |  |  |  |  | A |
| B | Rrc e | Rr e | Sra e Dec i? | Srl e | Ld bc, (*) | Ld de, (*) | Or (i?) Ld i?l, e | Ld sp, (*) | Otdr |  | Outd |  | Res *, (i?) Bit *, (i?) |  |  |  | B |
| C | Rrc h | Rr h | Sra h Inc i?l | Srl h | Ld c, i?h | Ld e, i?h | Le i?l, h | ld a, i?h | Adc a, i?h | Sbc i?h | Xor i?h | Cp i?h |  |  |  |  | C |
| D | Rrc l | Rr l | Sra l Dec i?l | Srl l | Reti Ld c, i?l | Ld e, i?l | Ld i?l, l | Ld a, i?l | Adc a, i?l | Sbc i?l | Xor i?l | Cp i?l |  |  |  |  | D |
| E | Rrc (hl) | Rr (hl) | Sra (hl) Ld i?l, * | Srl (hl) | ld c, (i?) | Im 2 ld e, (i?) | Ld i?l, (hl) Ld l, (i?) | Ld a, (i?) | Adc a, (i?) | Sbc a, (i?) | Xor (i?) | Cp (i?) |  |  | Additional details : 44 – Neg 45 – Retn 46 – ld b, (i?) |  | E |
| F | Rrc a | Rr a | Sra a | Srl a | Ld r, a | Ld a, r | Rld Ld i?l, a |  |  |  |  |  |  |  |  |  | F |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |  |

Theses Tables might contain a few errors, It's very hard to copy from one place to another without making any mistakes.  I've checked it quickly to make sure all the same instructions were pretty much in a block like they should be and it seems fine.  But still, if anyone finds a mistake in this, please notify me at "ti_chris@yahoo.com".