This is a program that calculates fast and easy the:

BUBLE Pressure	[y1; P]

DEW Pressure	[x1; P]

BUBLE Temperature	[y1; T]

DEW Temperature	[x1; T]

Azeotrope P		[x1 = y1; P]

Azeotrope T		[x1 = y1; T]

for binary compound mixes.

The program uses Antoine equation and has a 15 compound database, constants are in P_[kPa] and T_[°C]. The activity constant is calculated using Wilson’s equation; some constants are in the next table.

| System | Vi [cm3/mol] | a12 [cal/mol] | a21 [cal/mol] |
| --- | --- | --- | --- |
| Acetone (1) Water (2) | 74.05 18.07 | 291.27 | 1448.01 |
| Methanol (1) Water (2) | 40.73 18.07 | 107.38 | 469.55 |
| 1-Propanol (1) Water (2) | 75.14 18.07 | 775.48 | 1351.90 |
| Water (1) 1,4-Dioxane (2) | 18.07 85.71 | 1696.98 | -219.39 |
| Methanol (1) Benzene (2) | 40.73 89.41 | 1734.42 | 183.04 |
| Ethanol (1) Toluene (2) | 58.68 106.85 | 1556.45 | 210.52 |

You may find more at

Gmehling; et al. “Vapor-Liquid Equilibrium Data Collection”, Chemistry Data Series, vol. I, parts 1a, 1b, 2c, 2e, DECHEMA, Frankfurt/Main, 1981 – 1988.

Instructions:

simply execute s_vn\elv()

For TI-89, TI-89 titanium, TI-92 Plus, TI-Voyage 200.

Eric DiStefano

ej2x@hotmail.com